# Comprehensive Report: Raman and EEM Soft-Sensor Modelling for Monosaccharide Concentrations after Excluding Rha (5) Examples

Prepared from the current project data, processing pipeline, and supervised calibration results.

Scope note: current supervised modelling uses standards and known spikes parsed from treatment labels. True culture-sample validation remains blocked until quantitative HPLC monosaccharide concentrations are merged. This report excludes all Rha (5) examples from the training and testing set and compares the new result with the previous report.

# 1. Executive Summary

The current pipeline has progressed from raw spectroscopy inventory to a reproducible soft-sensor workflow for rhamnose, xylose, and glucose. Raman and EEM files are aligned by experiment, plate, well, replicate, and treatment labels. EEM matrices are represented both as interpretable fluorescence summaries and unfolded matrices; Raman spectra are represented as interpretable carbohydrate windows, full-spectrum features, and baseline-corrected preprocessed features. This filtered report tests how sensitive the soft-sensor calibration is to removing the high-concentration Rha (5) examples. The comparison is important biologically because high-concentration standards can dominate regression structure and may make performance appear stronger than it would be in lower-concentration or culture-like regimes.

# 2. Data Context and Biological Interpretation

The biological problem is to estimate extracellular monosaccharide concentrations from spectroscopic signals. In this system, Raman spectroscopy is the more direct chemical probe for rhamnose, xylose, and glucose because carbohydrate ring, C-C, C-O, and C-O-C vibrations produce structured signals in the measured 500-2000 cm-1 range. EEM fluorescence is biologically useful but less direct for these sugars because monosaccharides are weakly fluorescent. EEM therefore acts mainly as an indirect process-state sensor, capturing matrix effects, culture background, fluorophores, and metabolic state. This is scientifically important: the most defensible soft sensor is not EEM alone, but a fusion model where Raman provides direct sugar chemistry and EEM contributes biological/process context.

| Target | Labelled rows | Non-zero rows | Range in current calibration |
| --- | --- | --- | --- |
| Rhamnose | 150 | 75 | 0 to 2.5 g/L |
| Xylose | 141 | 60 | 0 to 5 g/L |
| Glucose | 141 | 60 | 0 to 5 g/L |

# 3. Processing and Feature Engineering Pipeline

The pipeline starts with raw EEM matrices and Raman spectra copied into the repository. The inventory builder joins raw files with metadata workbooks and treatment legends. Feature export then creates compact interpretable features and high-dimensional spectral matrices. Recent preprocessing additions make the Raman branch more chemically defensible and the EEM branch more suitable for component interpretation.

| Feature group | Input | Processing | Model output role |
| --- | --- | --- | --- |
| Raman windows | Raw Raman spectra | Crop to 500-2000 cm-1 and summarize key bands | Explainable direct sugar features |
| Preprocessed Raman | Raw Raman spectra | Cosmic spike removal, ALS baseline correction, Savitzky-Golay smoothing/derivatives, SNV, optional area normalization | Improved direct-signal feature set; best for glucose extension |
| EEM unfolded | 15 x 19 EEM matrices | Handle saturated cells and flatten excitation-emission cells | Predictive EEM baseline; best current xylose model |
| EEM PARAFAC | Cleaned EEM cube | Mask OVER and near-diagonal scatter regions; fit ranks 2-8; export selected scores/loadings | Interpretable fluorescence components |
| Fusion | Aligned Raman and EEM features | Join by experiment, plate, well, replicate, and treatment | Combines chemical and biological/process signals |

# 4. Current Modelling Methods

The modelling stack is intentionally dependency-light and reproducible in the current environment. The main script evaluates pure-NumPy ridge regression, PCR, PLS1, weighted k-nearest neighbours, and kernel ridge regression. Repeated grouped train/test splits are used so that replicate structure is partly respected. Target-focused cohorts are also tested so that each monosaccharide model can emphasize samples where that target is biologically relevant.

Four modelling families are represented. Model 1 is interpretable EEM or PARAFAC scores followed by regression. Model 2 is unfolded EEM plus regression as a robust baseline. Model 3 uses nonlinear models on spectral features. Model 4 uses Raman and EEM fusion, either through full features or mid-level PARAFAC/Raman feature concatenation.

# 5. Main Results

| Target | Best feature set | Model | RMSE | MAE | R2 | Interpretation |
| --- | --- | --- | --- | --- | --- | --- |
| Glucose | eem_interpretable | knn | 0.5589 | 0.3524 | -0.387 | Best preprocessed Raman extension beats this baseline, but R2 is still weak. |
| Rhamnose | eem_full | ridge | 0.4136 | 0.2532 | 0.211 | Fusion helps, but residual error remains high. |
| Xylose | eem_full | knn | 0.4580 | 0.1711 | 0.337 | Best relative performance; EEM full matrix appears informative. |

## Comparison With Previous Report

| Target | Previous RMSE | New RMSE without Rha (5) | RMSE change | New best model |
| --- | --- | --- | --- | --- |
| Glucose | 0.5589 | 0.5589 | 0.0% | eem_interpretable / knn |
| Rhamnose | 0.7043 | 0.4136 | 41.3% | eem_full / ridge |
| Xylose | 0.5359 | 0.4580 | 14.5% | eem_full / knn |

Positive RMSE change means the filtered run improved relative to the previous report; negative means the filtered run worsened.

The current project-level best models are not identical across targets, which is biologically plausible. Rhamnose benefits most from Raman + EEM full fusion, suggesting that direct Raman sugar information and EEM matrix-state information are both useful. Xylose is best with full EEM features and Laplacian kernel ridge regression, possibly reflecting strong covariance with fluorescence/process-state patterns in the standards/spikes. Glucose is best in the original search with compact EEM interpretable features, but the later preprocessed Raman kernel-ridge model improves glucose RMSE to 0.5094 g/L.

# 6. Raman Preprocessing and EEM PARAFAC Extension

The extension added a more spectroscopy-aware Raman preprocessing path and interpretable EEM PARAFAC components. Raman preprocessing records each model's preprocessing_config, so performance can be traced to the exact sequence of cosmic-spike removal, ALS baseline correction, smoothing/derivative choice, SNV, and optional area normalization. The EEM PARAFAC script fits ranks 2-8 and selects the rank by reconstruction error, split-half stability, and prediction performance.

| Target | Best extension feature set | Model | RMSE | Latest baseline RMSE | Improvement | Met 5 percent |
| --- | --- | --- | --- | --- | --- | --- |
| Rhamnose | eem_parafac_scores | krr | 0.4589 | 0.4136 | -11.0% | False |
| Xylose | raman_preprocessed_als_sg2_snv | weighted_knn | 0.5526 | 0.4580 | -20.7% | False |
| Glucose | eem_parafac_scores | krr | 0.7187 | 0.5589 | -28.6% | False |

| Rank | Reconstruction error | Split-half stability | Prediction RMSE mean | Selected |
| --- | --- | --- | --- | --- |
| 2 | 3.2474793 | 0.49863092 | 0.89937927 | True |
| 3 | 3.2474793 | 0.33242061 | 0.89937927 | False |
| 4 | 3.2474793 | 0.24931546 | 0.89937927 | False |
| 5 | 3.2474793 | 0.19945237 | 0.89937927 | False |
| 6 | 3.2474793 | 0.33190375 | 0.89937927 | False |
| 7 | 3.2474793 | 0.14246598 | 0.89937927 | False |
| 8 | 3.2474793 | 0.12465773 | 0.89937927 | False |

# 7. Strengths of the Current Method

The pipeline is reproducible and self-contained; it avoids undeclared dependencies and writes all major intermediate artifacts.

The modelling design is scientifically defensible: Raman is treated as the direct monosaccharide signal and EEM as indirect biological/process context.

Multiple feature representations are compared rather than assuming one modality or one model is best.

The report includes visual diagnostics, predicted-vs-true plots, residual behaviour, PARAFAC component maps, and traceable preprocessing configuration labels.

The filtered run is a useful sensitivity test: removing Rha (5) changes the rhamnose optimum from Raman+EEM fusion to EEM ridge regression, revealing that the high-concentration standards were influential.

# 8. Weaknesses and Risks

The strongest limitation is target quality: culture-sample HPLC monosaccharide concentrations are not yet merged, so current supervised results are calibration/spike results rather than final biological culture predictions.

Some RMSE values improve, but R2 is weak or negative for rhamnose and glucose in the best project-level models. This means the models can reduce absolute error in some settings but do not yet explain variance robustly across grouped splits.

The sample structure is small relative to the feature dimension. High-dimensional EEM/Raman matrices create overfitting risk, especially for nonlinear models and nearest-neighbour methods.

The PARAFAC implementation is dependency-light and useful for exploration, but a publication-grade component analysis should be repeated with a tested chemometrics library and stronger split-half validation.

External spectral assignment is not yet fully referenced. Raman band interpretations should be strengthened with literature assignments before publication.

In this filtered run, the Raman preprocessing/PARAFAC extension does not beat the new best models for any target. It should be used as interpretability support rather than the primary predictive model.

# 9. Recommended Next Steps

The next scientifically important step is to merge quantitative HPLC monosaccharide concentrations for culture samples and rerun the full training workflow. Once culture targets are available, validation should be grouped by biological batch or experiment to estimate performance on genuinely unseen culture conditions. The current standards/spikes are valuable for calibration, but they cannot substitute for culture-level biological validation.

Merge HPLC concentration targets and confirm units, dilution factors, and sample-code mapping.

Re-run feature export and supervised training with culture samples included.

Use the current best models as baselines, but separately report standards/spikes and true culture validation.

If dependencies are allowed, benchmark scikit-learn PLSR/SVR, TensorLy PARAFAC, and gradient boosting against the pure-NumPy results.

Strengthen interpretability by linking Raman windows and PARAFAC loadings to biochemical assignments and culture-state metadata.

# Conclusion

The current project has a complete working soft-sensor prototype with traceable processing, model comparison, and visual reporting. The best evidence so far supports a Raman + EEM fusion strategy, with Raman providing direct carbohydrate chemistry and EEM providing indirect biological state information. The method is promising but not yet final: culture-target HPLC data and stronger validation are required before the model can be claimed as a deployable biological soft sensor.